IOT Lab Manual

1.This Code creates a back-and-forth LED pattern using digitalWrite() and delay() functions. The forward sequence (1-5) followed by  reverse sequence (4-2) creates a continuous bouncing effect.

// LED Pin Declarations

int LED1 = 5;

int LED2 = 6;

int LED3 = 7;

int LED4 = 8;

int LED5 = 9;

int del = 100;

void setup() {

pinMode(LED1, OUTPUT);

pinMode(LED2, OUTPUT);

pinMode(LED3, OUTPUT);

pinMode(LED4, OUTPUT);

pinMode(LED5, OUTPUT);

}

void loop() {

digitalWrite(LED1, HIGH);

delay(del);

digitalWrite(LED1, LOW);

digitalWrite(LED2, HIGH);

delay(del);

digitalWrite(LED2, LOW);

digitalWrite(LED3, HIGH);

delay(del);

digitalWrite(LED3, LOW);

digitalWrite(LED4, HIGH);

delay(del);

digitalWrite(LED4, LOW);

digitalWrite(LED5, HIGH);

delay(del);

digitalWrite(LED5, LOW);

digitalWrite(LED4, HIGH);

delay(del);

digitalWrite(LED4, LOW);

digitalWrite(LED3, HIGH);

delay(del);

digitalWrite(LED3, LOW);

digitalWrite(LED2, HIGH);

delay(del);

digitalWrite(LED2, LOW);

}

2. This code controls a relay v=by sending High/Low signals to digital pin 4. When HIGH, the relay activates (closes circuit), when LOW, the relay deactivates (open circuit).

void setup() {

pinMode(4, OUTPUT);

}

void loop() {

digitalWrite(4, HIGH);

delay(1000); // Wait for 1 second

digitalWrite(4, LOW);

delay(1000); // Wait for 1 second

}

3. This code measures distance using ultrasonic sensor time-of-fight calculation. When distance is <_10cm, it triggers the buzzer alarm and displays security status on serial Monitor.

// Intrusion Detection System - Pin Declarations

int trigger_pin = 2;

int echo_pin = 3;

int buzzer_pin = 10;

int time;

int distance;

void setup() {

Serial.begin(9600);

pinMode(trigger_pin, OUTPUT);

pinMode(echo_pin, INPUT);

pinMode(buzzer_pin, OUTPUT);

}

void loop() {

// Send ultrasonic pulse

digitalWrite(trigger_pin, HIGH);

delayMicroseconds(10);

digitalWrite(trigger_pin, LOW);

// Measure pulse duration and calculate distance

time = pulseIn(echo_pin, HIGH);

distance = (time * 0.034) / 2;

// Check for intrusion (distance ≤ 10cm)

if (distance <= 10) {

Serial.print("Door Open");

Serial.print(" Distance=");

Serial.println(distance);

digitalWrite(buzzer_pin, HIGH);

} else {

Serial.print("Door Closed");

Serial.print(" Distance=");

Serial.println(distance);

digitalWrite(buzzer_pin, LOW);

}

delay(500);

}

4. This code uses digitalWrite() to turn the motor ON and OFF. Pin 9 is set as OUTPUT to control the transistor base, which in turn controls the motor operation.

// DC Motor Control Pin

int pin = 9;

void setup() {

// Set pin 9 as output for motor control

pinMode(pin, OUTPUT);

}

void loop() {

// Turn motor ON

digitalWrite(pin, HIGH);

delay(1000); // Wait for 1000 milliseconds

// Turn motor OFF

digitalWrite(pin, LOW);

delay(1000); // Wait for 1000 milliseconds

}

5.This code reads the LDR sensor value using analogRead() and compares it with a threshold (300). If the light level is below the threshold (dark), the LED turns ON; otherwise, it turns OFF.

// Smart Street Light System using LDR

int led = 10;      // LED connected to digital pin 10

int ldr = A0;      // LDR connected to analog pin A0

void setup() {

Serial.begin(9600);        // Initialize serial communication

pinMode(led, OUTPUT);       // Set LED pin as output

pinMode(ldr, INPUT);        // Set LDR pin as input

}

void loop() {

int ldrStatus = analogRead(ldr);   // Read LDR sensor value

// Print sensor value for debugging

Serial.print("LDR Value: ");

Serial.println(ldrStatus);

// If light level is low (dark), turn ON the street light

if (ldrStatus<= 300) {

digitalWrite(led, HIGH);    // Turn ON LED (Street Light)

Serial.println("Street Light: ON (Dark detected)");

} else {

digitalWrite(led, LOW);     // Turn OFF LED (Street Light)

Serial.println("Street Light: OFF (Sufficient light)");

}

delay(500);  // Wait for half second before next reading

}

6.This code integrates ultrasonic detection with moisture sensing to automatically classify and sort waste. The system uses servo motors for mechanical sorting and LCD displays for user feedback.

// Smart Waste Classification System

#include <Servo.h>

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

const int trigPin = 9;

const int echoPin = 10;

const int soilMoisturePin = A0;

Servo myservo;

LiquidCrystal_I2C lcd(0x27, 16, 2);

long duration;

int distance;

// Threshold values for classification

const int distanceThreshold = 100; // cm

const int moistureThreshold = 500; // Adjust as needed

void setup() {

Serial.begin(9600);

pinMode(trigPin, OUTPUT);

pinMode(echoPin, INPUT);

pinMode(soilMoisturePin, INPUT);

myservo.attach(11);

// Initialize LCD

lcd.init();

lcd.backlight();

lcd.setCursor(0,0);

lcd.print("Wet/Dry Segregator");

delay(2000);

}

void loop() {

// Ultrasonic sensor measurement

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

duration = pulseIn(echoPin, HIGH, 30000);

distance = duration * 0.034 / 2;

// Read moisture level

int moistureValue = analogRead(soilMoisturePin);

// Display readings on LCD

lcd.clear();

lcd.setCursor(0,0);

lcd.print("Dist: ");

lcd.print(distance);

lcd.setCursor(0,1);

lcd.print("Moist: ");

lcd.print(moistureValue);

// Object detection and classification

if (distance > 0 && distance <distanceThreshold) {

if (moistureValue>moistureThreshold) {

// Wet waste detected

myservo.write(0);

lcd.setCursor(10,0);

lcd.print("Wet");

Serial.println("Wet object detected!");

} else {

// Dry waste detected

myservo.write(90);

lcd.setCursor(10,0);

lcd.print("Dry");

Serial.println("Dry object detected!");

}

} else {

Serial.println("No object detected within threshold!");

}

delay(1000);

}

7. This code reads analog values from the pH sensor, converts them to voltage, then calculates pH using calibration formula. Result are displayed on LCD with substance classification.

// Include LCD library

#include <LiquidCrystal.h>

// Initialize the LCD (RS, E, D4, D5, D6, D7)

LiquidCrystallcd(7, 8, 9, 10, 11, 12);

#define pH_PIN A0 // Potentiometer as pH Sensor

void setup() {

lcd.begin(16, 2);

Serial.begin(9600);

lcd.print("Simulating pH");

delay(2000);

lcd.clear();

}

void loop() {

int sensorValue = analogRead(pH_PIN); // Read from potentiometer

float voltage = sensorValue * (5.0 / 1023.0); // Convert to voltage

float pHValue = voltage * 2.8; // Approximate pH conversion (0-14 range)

lcd.setCursor(0, 0);

lcd.print("pH Value: ");

lcd.print(pHValue);

lcd.setCursor(0, 1);

if (pHValue< 6.5) {

lcd.print("Acidic (Lime)");

} else if (pHValue> 7.5) {

lcd.print("Basic (Soap)");

} else {

lcd.print("Neutral (Water)");

}

Serial.print("pH Value: ");

Serial.println(pHValue);

delay(2000); // Wait for next reading

}

8. This code reads analog values from the MQ-2 gas sensor and displays the gas level readings on the serial monitor. Higher vales indicate higher gas concentrations.

// Gas Sensor Pin Declaration

int gasSensor = A1;

void setup() {

// Initialize the gas sensor pin as input

pinMode(A1, INPUT);

// Initialize serial communication at 9600 baud rate

Serial.begin(9600);

// Display startup message

Serial.println("Gas Detection System Initialized");

Serial.println("Warming up sensor...");

delay(20000); // 20 second warm-up time

Serial.println("Ready to detect gas!");

}

void loop() {

// Read the analog value from gas sensor

int sensorValue = analogRead(gasSensor);

// Display gas level on Serial Monitor

Serial.print("GAS LEVEL: ");

Serial.println(sensorValue);

// Check for gas leakage threshold

if (sensorValue> 400) {

Serial.println("⚠️ WARNING: Gas leakage detected!");

} else if (sensorValue> 200) {

Serial.println("⚡ CAUTION: Elevated gas levels");

} else {

Serial.println("✅ Normal gas levels");

}

// Wait 1 second before next reading

delay(1000);

}